  - DELAYED EGRESS LOCKS

    - Delayed Egress Controller:

** NOTE TO SPECIFIER **   Delete model not required.

      - Product: Model 101-DE - Wall mount controller with keypad control and reset as manufactured by SDC Security Door Controls.

      - Product: Model 101-KDE - Wall mount controller with both keypad and key switch control and reset (less cylinder) as manufactured by SDC Security Door Controls.

        - Inputs: Voltage input: Auto sensing 12/24 VDC; Reset input: NO, Dry; REX input: NO, Dry; Trigger: N.C or N.O., dry.

        - Monitoring Outputs: Alarm Output: 1 Amp at 30 VDC SPDT, dry; Locked Output: 1 Amp at 30 VDC SPDT, dry.

      - Operation: NA - NFPA, UBC, IBC, IFC,. (1 or 2 sec Nuisance. 15 or 30 sec Delay)

      - Operation ND -  NFPA,, UBC, IFC, IBC, SBC. (0/1 sec Nuisance. 15 sec Fixed Delay)

      - Operation NH - NFPA,, UBC, IFC, IBC, SBC. (0/1 sec Nuisance. 30 sec Fixed Delay)

      - Operation NC - NFPA,, UBC, IFC, IBC, SBC, CBC OSPHD. (0/1 sec Nuisance. 15 sec Fixed Delay. Manual Reset)

      - Operation BC – BOCA and Chicago Code. (0 sec Nuisance. 15 sec fixed Delay. 30 sec Relock)

      - Operation BH – BOCA Code. (0/1 sec Nuisance. 30 sec fixed Delay. 30 sec Relock)

      - Operation: BD – BOCA Code. 0/1 sec Nuisance. 15 sec fixed Delay. 30 sec Relock)

** NOTE TO SPECIFIER **  Finish: Delete finish not required for EMlock. Plated finishes are special order.

      - Finish: V - 628 Aluminum.

      - Finish: X - Dark Bronze powder coat (313/613 equal).

      - Finish: Y - 335 Black Anodized.

      - Specify EMlock to be used

        - 1511DEV – Single, 1650lbs with trigger

        - 1571DEV – Single, 1200lbs with trigger

        - 1581DEV – Single, 650lbs with trigger

        - 1575DEU – Outdoor with trigger

        - 1576DEU – Outdoor face drilled with trigger

        - 1576DE-ZB – Inswing mounting bracket for 1576DEU

** NOTE TO SPECIFIER **   Delete options not required. Specify two for tandem operation of options D, B and A.

      - Accessory: D - Door Position Status. Provides remote monitoring.(DPS not available with 1581, 1575, 1576)

      - Accessory: B - Magnetic Bond Alert Sensor.

      - Accessory: VI - One Language or Bilingual - Spanish, or English and Spanish, or English and French verbal notification.