  - ELECTRIC BOLT LOCKS

** NOTE TO SPECIFIER **  The Spacesaver®, first designed and patented by SDC, is a fundamental innovation in electric locking technology for access control applications. The stainless steel bolt projects at right angles to the lock mechanism, allowing installation of Spacesaver® locks, by means of a simple cutout, in virtually any standard 1.5 inches frame. , or in most door lock stiles. Delete if not required.

    - Bolt Locks, Mortise:

      - Spacesaver Series as manufactured by SDC Security Door Controls. The stainless steel bolt projects at right angles to the lock mechanism minimizing installation space requirements.

      - Auto relock switch, 12/24 VDC operation.

** NOTE TO SPECIFIER **  Delete model not required.

      - Product: Model Spacesaver 1091AI - Failsafe Bolt Lock as manufactured by SDC Security Door Controls. Dual Voltage Coil: 900/450 mA @ 12/24 VDC.

      - Product: Model Spacesaver 1091ADLI - Failsafe Deadlocking Bolt Lock as manufactured by SDC Security Door Controls. Dual Voltage Coil: 900/450 mA @ 12/24 VDC.

      - Product: Model Spacesaver 1291AH - Failsecure Bolt Lock as manufactured by SDC Security Door Controls. Dual Voltage Coil: 900/450 mA @ 12/24 VDC. Note - Will not unlock without power

      - Finish: D - 606 Dull Brass.

      - Finish: F - 611 Bright Bronze.

      - Finish: G - 612 Dull Bronze.

      - Finish: H - 613 Oil Rubbed Bronze.

      - Finish: P - 625 Bright Chrome.

      - Finish: Q - 626 Dull Chrome.

      - Finish: U - 630 Satin Stainless Steel.

** NOTE TO SPECIFIER **  Delete option not required.

      - PR-1000 – Power regulator and heat reducer for 1091, 1291, 1190, FS23

      - Accessory: B - Magnetic Bolt Status Switch. SPDT 0.25 Amp @ 30VDC. Indicates bolt locked and unlocked.

      - Accessory: D - Mechanical Door Position Switch. SPDT 5 Amps at 30VDC. Integral to the auto relock ball switch assembly. Indicates, door closed and door open. Recommended for minimum security applications only.

      - Accessory: BR64XL - Rectifier for AC operation.