  - MAGNETIC LOCKS AND GATE LOCKS

    - Electromagnetic Locks, Surface Mount:

      - ANSI Grade 1 (1650 lb (748 Kg) holding force). 12/24 VDC (700 mA/350 mA).

** NOTE TO SPECIFIER **  Delete model not required.

        - Single Door: Model 1511 EMLock as manufactured by SDC Security Door Controls.

        - Double Door: Model 1512  EMLock as manufactured by SDC Security Door Controls.

      - ANSI Grade 1 (1200 lb (544 Kg) holding force). 12/24 VDC (250 mA/125 mA).

** NOTE TO SPECIFIER **  Delete model not required.

        - Single Door: Model 1571 EMLock as manufactured by SDC Security Door Controls.

        - Double Door: Model 1572 EMLock as manufactured by SDC Security Door Controls.

      - ANSI Grade 1 (650 lb (295 Kg) holding force). 12/24 VDC (500 mA/250 mA).

** NOTE TO SPECIFIER **  Delete model not required.

        - Single Door: Model 1581 EMLock as manufactured by SDC Security Door Controls.

        - Double Door: Model 1582 EMLock as manufactured by SDC Security Door Controls.

** NOTE TO SPECIFIER **  Delete if not required.

      - Provide Universal adjustable glass door bracket.

        - HDB1 – Single glass door bracket

        - HDB2 – Double glass door bracket

** NOTE TO SPECIFIER **  Delete if not required.

      - Provide Adjustable universal top jamb bracket.

        - TJI – Single bracket, 1511, 1571

        - TJ2 – Double bracket, 1512,1572

        - TJ81 – Single Bracket, 1581

        - TJ82 – Double bracket, 1582

** NOTE TO SPECIFIER **  Finish V is standard. Delete finish not required. Plated finishes are special order.

      - Finish: V -  628 Aluminum.

      - Finish: X -  Dark Bronze powder coat (313/613 equal).

      - Finish: Y -  335 Black Anodized.

      - Plated Finish: Q - 626 Dull Chrome.

      - Powder Coat Finish: C - 605 Bright Brass

      - Plated Finish: P - 625 Bright Chrome.

** NOTE TO SPECIFIER **  Monitoring and Timer Options. Multiple options possible. Delete options not required.

      - Accessory: A - Anti-tamper Sensor: Indicates access cover removal. SPDT Dry, 1 Amp at 30VDC.

      - Accessory: D - Door position sensor.

      - Accessory: B - Magnetic bond sensor.

      - Accessory: T - Adjustable 1-30 sec. timer. (N/A for 1581, 1582)

      - Accessory: L - Bi-Color LED.

** NOTE TO SPECIFIER **  Refer to manufacturer's literature. Delete mounting options not required.

      - Housing: Architectural housing. (1510 and 1570 Emlocks)

      - Bracket: Header mounting bracket.

      - Bracket: Universal and concrete mounting brackets.

        - UF11, or UF12, or UF81, or UF82

      - Bracket: AR Armature mounting bracket.

        - AR11Y – Single 1511 or 1571. Black

        - AR11YD – Single 1511 or 1571 with DPS. Black

      - Brackets: Filler plates - FP or Angle brackets - AB.

      - Bracket: Door coordinator mounting brackets.

        - DC-1 -  Single for 1511 or 1571

        - DC-2 – Double for 1512 or 1572