  - ELECTRIC BOLT LOCKS

** NOTE TO SPECIFIER **  The Spacesaver®, first designed and patented by SDC, is a fundamental innovation in electric locking technology for access control applications. The stainless steel bolt projects at right angles to the lock mechanism, allowing installation of Spacesaver® locks, by means of a simple cutout, in virtually any standard 1.5 inches frame. , or in most door lock stiles. Delete if not required.

    - Bolt Locks, Surface Mount:

      - Lock Dimensions: 6 inches L by 2 inches W by 2 inches H (152.4 by 50.8 by 50.8 mm).

      - Bolt: Solid Stainless Steel 0.5 inch diameter by 0.5 inch throw (12.7 by 12.7 mm).

      - Dual Voltage Coil: 0.9 Amp at 12 VDC continuous, 0.45 Amp at 24 VDC continuous.

** NOTE TO SPECIFIER **  Delete model not required.

      - Product: Model 180AI - Surface mount bolt lock, failsafe, with auto relock switch and power regulator.

      - Product: Model 280AH - Surface mount bolt lock, failsecure with auto relock switch and power regulator. Note – will not unlock without power

** NOTE TO SPECIFIER **  Delete finish not required.

      - Finish: V - 628 Aluminum.

** NOTE TO SPECIFIER **  Delete options not required.

      - Accessory: D – MC-4 External door position sensor, SPDT