  - EXIT SWITCHES

** NOTE TO SPECIFIER **  430 Series. Delete if not required.

      - 430 Series - 1.5 inches (38 mm) Green Mushroom Button (Red optional)

        - 430 Series – Single gang box mounting

        - 430N Series – Narrow mullion mounting

** NOTE TO SPECIFIER **  Available Model for function indicated. Delete model function not required.

        - Product: Model 431C/431CN - AA (On/Off) NC SPST. (No Sign).

        - Product: Model 431O/431ON - AA (On/Off) NO SPST. (No Sign).

        - Product: Model 432C/432CN - Momentary NC SPST. (EXIT).

        - Product: Model 432O/432ON - Momentary NO SPST. (EXIT).

        - Product: Model 433 – Double Gang, Momentary with Timer Module, Adjustable 1-60 seconds 12/24V AC/DC, SPDT 2A contact. (EXIT).

        - Product: Model 434/434N - Momentary DPST. (EXIT).

        - Product: Model 435/435N - AA (On/Off) DPST. (No Sign).

        - Contact: 6 Amp at 30 VDC.

        - Wire Connection: Screw Terminals.

** NOTE TO SPECIFIER **  The SDC 432KUR latches when depressed and must be reset with the built-in-key. Applications include alarm, panic alarm and emergency door release where systems are restored by authorized personnel only.

        - Product: Model 432KUR/432KNUR - Latching/ key reset DPST (No Sign).