  - ELECTRIC STRIKES

    - Universal Commercial Duty Electric Strikes:

      - Up to 3/4 inch (19 mm) latch bolts. 1/4 inch (6 mm) horizontal alignment adjustment. Low profile 1-1/8 inches (29 mm) deep.

      - Field selectable failsafe/failsecure. Field selectable 170mA/280mA at 12VAC/DC or 85mA/140mA at 24VAC/DC. Latch Position Status: SPDT, Dry 3 Amp at 30 V. Keeper Closed and Locked Status: SPDT, Dry 3 Amp at 30 V.

** NOTE TO SPECIFIER **  Delete model not required.

      - Product: Model 45-4SU - 4.875 inches (124 mm) square corners as manufactured by SDC Security Door Controls.

      - Product: Model 45-6SV - 6.875 inches (175 mm) square corners as manufactured by SDC Security Door Controls.

      - Product: Model 45-4RV - 4.875 inches (124 mm) radius corners as manufactured by SDC Security Door Controls.

      - Product: Model 45-6RV - 6.875 inches (175 mm) radius corners as manufactured by SDC Security Door Controls.

      - Product: Model 45-7RV – 7.937 inches (202 mm) radius corners as manufactured by SDC Security Door Controls.

      - ** NOTE TO SPECIFIER ** Finish: Delete finish not required.

      - Finish: U - 630 Dull Stainless Steel.

      - Finish: V - 628 Dull Aluminum.

      - Finish: Y - 335 Black Anodized.

      - Finish: D - 606 Dull Brass.

      - Finish: H - 613 Oil Rubbed Bronze.

** NOTE TO SPECIFIER **  Delete Buzzer and Keeper options if not required.

      - Provide Buzzer RMB 12/24 V AC/DC operation.

      - Provide Keeper Open or Closed status Option K SPDT, Dry 3 Amp at 30 V.