  - DELAYED EGRESS LOCKS

    - Annunciators:

** NOTE TO SPECIFIER **   Compatible with all Exit Check systems, the 101 series annunciators provide door status at a remote location such as a nurse's station, guard station or office. The tri-color LED provides precise delay mode status.

      - Annunciator:

        - Faceplate: 20 gauge stainless steel.

        - Key Switch: 6 Amp at 30 VDC maximum.

        - Push Switch: 5 Amp at 30 VDC maximum.

        - LED: Tri-color, 20 mA at 12/24VDC. Green -  Secure, Amber - Alarm/door locked, Red - Alarm/door unlocked.

** NOTE TO SPECIFIER **   Delete model not required.

      - Product: Annunciator Model 101-1A - Single Door Annunciator as manufactured by SDC Security Door Controls:

        - Audible alarm.

        - Tri-color LED status indicator.

      - Product: Annunciator Model 101-4AM - Four Door Annunciator as manufactured by SDC Security Door Controls:

        - Audible alarm.

        - Alarm mute switch.

        - Four tri-color LED status indicators.

      - Product: Annunciator Model 101-1AK - Single Door Annunciator with Key Control as manufactured by SDC Security Door Controls:

        - Dual function key switch for bypass (on/off) and reset. Key cylinder not included.

        - Audible alarm.

        - Tri-color LED status indicator.

        - Application required adjacent to the door.

      - Product: Annunciator Model 101-PAM - Single Door Annunciator with Push Switch Control as manufactured by SDC Security Door Controls:

        - Three push switches: momentary access, bypass (on/off) and audible mute.

        - Audible alarm.

        - Tri-color LED status indicator.

      - Product: Model 101-SP - External 15 watt speaker connects directly to 1511S or 1511T for enhanced decibels of onsite or remote voice message annunciation. Speaker driver may be provided by others for extreme decibel requirements.

      - Product: Model 101-SPSN - External speaker and piezo sounder connects directly to 101-DE or 101-KDE. Duplicates ExitCheck voice message and alarm.