  - EXIT SWITCHES

    - Exit Switches:

      - Product as manufactured by SDC Security Door Controls.

** NOTE TO SPECIFIER **  410 & 420 Series. Delete if not required.

      - Illuminated switch button. The high impact resistant material resists abuse. The "PUSH to EXIT" sign complies with NFPA requirements.

        - Contact: SPDT, 5 Amp at 30 VDC.

        - Timer: SPDT 2 Amp at 12/24 VDC.

        - Wire Leads: 6 inches (152 mm), 20 Gauge.

        - 420 Series – Single gang box mounting

        - 410N Series – Narrow Mullion mounting

** NOTE TO SPECIFIER **  Available Model for function indicated. Delete model function not required.

        - Product: Model 422/412N - Momentary SPDT (PUSH TO EXIT).

        - Product: Model 423/413N - Integrated electronic timer, adjustable 1-60 sec, 12/24 VDC, DPDT 2 Amp contact (PUSH TO EXIT).

        - Product: Model 423P/413PN - Integrated pneumatic timer, adjustable 1-180 sec. 12/24 VDC,  DPST 3 Amp contact (PUSH TO EXIT).

        - Product: Model 424/414N - Momentary DPDT (PUSH TO EXIT).

        - Product: Model 425/415N - AA (On/Off) DPDT (No Sign).

        - Product: Model 422A - Momentary SPDT. 2 inches Button – Blue ADA Illuminated button only.

        - Product: Model 423A - Integrated electronic timer, adjustable 1-60 sec, 12/24VDC, DPDT 2Amp contact. 2 inches (51 mm) Button – Blue ADA Illuminated button only.

** NOTE TO SPECIFIER **  Code Compliant Manual Releasing Devices and Sensors 423M and 413MN exit switches used together with the MD31D PIR sensor comply with manual releasing device and sensor device requirements for Access-Controlled Egress Doors per the following codes: NFPA 101 Life Safety Code - 7.2.1.6.2; California Building Code. Delete if not required.

        - Model 423MU/413MNU - Integrated electronic timer, fixed 30 sec timer, 12/24VDC, DPDT 2Amp contact. (PUSH TO EXIT).

** NOTE TO SPECIFIER **  430 Series. Delete if not required.

      - 430 Series - 1.5 inches (38 mm) Green Mushroom Button (Red optional)

        - 430 Series – Single gang box mounting

        - 430N Series – Narrow mullion mounting

** NOTE TO SPECIFIER **  Available Model for function indicated. Delete model function not required.

        - Product: Model 431C/431CN - AA (On/Off) NC SPST. (No Sign).

        - Product: Model 431O/431ON - AA (On/Off) NO SPST. (No Sign).

        - Product: Model 432C/432CN - Momentary NC SPST. (EXIT).

        - Product: Model 432O/432ON - Momentary NO SPST. (EXIT).

        - Product: Model 433 – Double Gang, Momentary with Timer Module, Adjustable 1-60 seconds 12/24V AC/DC, SPDT 2A contact. (EXIT).

        - Product: Model 434/434N - Momentary DPST. (EXIT).

        - Product: Model 435/435N - AA (On/Off) DPST. (No Sign).

        - Contact: 6 Amp at 30 VDC.

        - Wire Connection: Screw Terminals.

** NOTE TO SPECIFIER **  The SDC 432KUR latches when depressed and must be reset with the built-in-key. Applications include alarm, panic alarm and emergency door release where systems are restored by authorized personnel only.

        - Product: Model 432KUR/432KNUR - Latching/ key reset DPST (No Sign).

** NOTE TO SPECIFIER **  440 Series. Delete if not required.

      - 440 Series - 2-5/8 inches (67 mm) Red Mushroom Button. (Green optional)

        - Contact: 6 Amp at 30 VDC.

        - Wire Connection: Screw Terminals.

        - 441 thru 444 – Double gang box mounting

        - 446 – Single gang box mounting

** NOTE TO SPECIFIER **  Available Model for function indicated. Delete model function not required.

        - Product: Model 441 - AA (On/Off) DPST. (No Sign).

        - Product: Model 442 - Momentary DPST. (EXIT).

        - Product: Model 443 - Momentary with Timer Module, Adjustable 1-60 seconds 12/24V AC/DC, SPDT 2A contact. (EXIT).

        - Product: Model 444 - Momentary 2-DPST. (EXIT).

        - Product: Model 446 - Momentary DPST. (EXIT).

** NOTE TO SPECIFIER **  Finishes for all series switches. Finish: U standard. Delete finishes not required.

      - Finish: U - Stainless Steel.

      - Finish: C - 605 Bright Brass.

      - Finish: D - 606 Dull Brass.

      - Finish: H - 613 Oil Rubbed Bronze.

      - Finish: P - 625 Bright Chrome.

** NOTE TO SPECIFIER **  Options for all series switches. L1G, L1R, L2, LT select only one.

      - Accessory: L1G - 1 Green LED.

      - Accessory: LT - Tri-color LED - Red, Green, Amber.

      - Accessory: L1R - 1 Red LED.

      - Accessory: L2 - 1 Red, 1 Green LED.

      - Accessory: E - Add Exit Sign to AA Function Switch.

      - Accessory: G - Green Push Button.

      - Accessory: R - Red Push Button.