  - ELECTRIC BOLT LOCKS

** NOTE TO SPECIFIER **  The Spacesaver®, first designed and patented by SDC, is a fundamental innovation in electric locking technology for access control applications. The stainless steel bolt projects at right angles to the lock mechanism, allowing installation of Spacesaver® locks, by means of a simple cutout, in virtually any standard 1.5 inches frame. , or in most door lock stiles. Delete if not required.

** NOTE TO SPECIFIER **  Conventional failsafe locking bolts rely solely on a spring to retract the bolt while the FS23M bolt will unlock with or without the assistance of the spring. This is the only electric bolt lock listed by the California State Fire Marshal for locking of perimeter entrance and emergency exit doors after-hours.

      - Product: Model Spacesaver FS23MI - Bolt Lock, Narrow, Dual Failsafe, CSFM Listed as manufactured by SDC Security Door Controls. Dual Voltage Coil: 900/450 mA @ 12/24 VDC.

** NOTE TO SPECIFIER **  Aluminum Finish is standard. Delete finish not required.

      - Finish: V - 628 Aluminum.

      - Finish: Y - 335 Black Anodized.

      - Finish: C - 605 Bright Brass.

      - Finish: D - 606 Dull Brass.

      - Finish: F - 611 Bright Bronze.

      - Finish: G - 612 Dull Bronze.

      - Finish: H - 613 Oil Rubbed Bronze.

      - Finish: P - 625 Bright Chrome.

      - Finish: Q - 626 Dull Chrome.

      - Finish: U - 630 Satin Stainless Steel.

** NOTE TO SPECIFIER **  Delete option not required.

      - PR-1000 – Power regulator and heat reducer for 1091, 1291, 1190, FS23

      - Accessory: B - Magnetic Bolt Status Switch. SPDT 0.25 Amp @ 30VDC. Indicates bolt locked and unlocked.

      - Accessory: D - Mechanical Door Position Switch. SPDT 5 Amps at 30VDC. Integral to the auto relock ball switch assembly. Indicates, door closed and door open. Recommended for minimum security applications only.

      - Accessory: BR64XL - Rectifier for AC operation.