  - ELECTRIFIED MORTISE LOCKSETS

    - Power Transfer Hinges (PTH):

      - UL 10B listed for 3 hour fire doors. 4.5 by 4.5 five knuckle standard weight. 5 feet (1524 mm) cable as manufactured by SDC Security Door Controls.

** NOTE TO SPECIFIER **  Delete model not required.

      - Product: Model PTH-4 - Four wires, 1 Amp at 12/24 VDC per pair as manufactured by SDC Security Door Controls.

      - Product: Model PTH-2+4 - 16 Amp inrush, 6 wire. 1 pair 16 Amp inrush, 4 Amp continuous.  2 pair 1 Amp @ 12/24V as manufactured by SDC Security Door Controls.

      - Product: Model PTH-10 - Ten Wires, 1 Amp at 12/24/VDC as manufactured by SDC Security Door Controls.

** NOTE TO SPECIFIER **  Finish Q standard. Delete finish not required.

      - Steel Finish: Q - 626 Dull Chrome.

      - Steel Finish: H - 613 Oil Rubbed Bronze.

      - Steel Finish: C - 605 Bright Brass.

      - Steel Finish: P - 625 Bright Chrome.

** NOTE TO SPECIFIER **  Delete if not required.

      - Option: DPS - Door position switch. SPDT, 250mA at 30 VDC.